Recommendation of the supervisor on Ph.D. thesis of Shi Hu MSc.

Thesis title:  Complex Analysis of EMI Shielding Fabrics

Doctoral student Shi Hu submitted a dissertation for defense in the form of annotated published scientific articles in high-impact journals. The work is focused on textiles shielding electromagnetic radiation, which nowadays are increasingly used both in the field of clothing and technical textiles. Thin (thickness 0.04 to 0.1 mm) thermally bonded, porous polyester non-woven fabrics with deposition of copper particles (MEFTEX Bochemia s.r.o.) are studied. Unlike other solutions, these textiles do not have a compact metallized layer on the surface, but copper particles are fixed on the surface of the fibers, so that porosity and air permeability are maintained on the one hand and flexibility on the other. At the same time, thanks to the perpendicular laying of 2 layers of polyester multifilaments on top of each other (similar to fabrics) and the point thermal bonding, there is low deformability.

The doctoral student observed the influence of the parameters of these simple and layered textile structures on porosity, air permeability and effectiveness of electromagnetic shielding.  Significant increase in the effectiveness of electromagnetic shielding was achieved through layering. Furthermore, the student managed to successfully simulate the effectiveness of the electromagnetic shielding of the mentioned structures using the finite element method in the ANSYS system, where mostly insignificant, but also significant differences between the simulated results and real measurements were found. The main problem was the inhomogeneity in the pore size distribution. The doctoral student successfully proposed a new mathematical model for predicting the effectiveness of electromagnetic shielding based on the transformation of the nonwoven fabric structure into a woven fabric structure with the same porosity, thickness and electrical conductivity.

The general problem of coated textiles is reduced resistance in washing, so the next task was to improve resistance in washing while maintaining the required effectiveness of electromagnetic shielding and air permeability. This was achieved by applying chemical vapor deposition of Parylene-C.

Overall, I evaluate the PhD student's dissertation as comprehensive, dealing with the necessary problems, making it possible to construct optimal textile structures with increased efficiency of electromagnetic shielding for various fields of application in practice.

The language proficiency exhibited in the thesis is commendable and satisfies the standards expected at the doctoral level. His exceptional abilities are evidenced by his publication record in journals with high-impact factors. Throughout his research tenure at TUL, he has promoted his findings through the publication of 14 papers in journals with high-impact factors (13 papers Q1, Q2), 5 book chapters, and 18 articles in conference proceedings. Throughout his academic pursuits, he demonstrated a high level of diligence and competency.

The findings of the dissertation are valuable, innovative, and readily applicable in practical settings. Thus, it is highly recommended that the thesis be presented for the final doctoral defense.

During the plagiarism check, 5% similarities was found with V. Tunáková's habilitation, which used the same measurement method, but this information does not affect the original results of student Shi Hu's dissertation (it is a description of the standard measurement method, citations from the standard, citations of some publications). The same is the case in other similarities of 2.6% and smaller.

Liberec January 20, 2025					Doc Dr. Ing. Dana Křemenáková

Supervisor